// Behavioral description of four-to-one line multiplexer

// Verilog 2001, 2005 port syntax

module mux_4x1_beh

( output reg	m_out,

input	in_0, in_1, in_2, in_3,

input [1: 0]	select

);

always @ (in_0, in_1, in_2, in_3, select) 	// Verilog 2001, 2005, SV syntax

case (select)

2’b00:	m_out  in_0;

2’b01:	m_out  in_1;

2’b10:	m_out  in_2;

2’b11:	m_out  in_3;

endcase

endmodule