// Mux with three-state output

module mux_tri (m_out, A, B, select);

output	m_out;

input	A, B, select;

tri			m_out;

bufif1 (m_out, A, select);

bufif0 (m_out, B, select);

endmodule