entity or2_gate is

port (w: out Std_Logic; x, y: in Std_Logic);

end or2_gate;

architecture Dataflow of or2_gate is

begin

w <= x or y;

end Dataflow;

entity or3_gate is

port (w: out Std_Logic; x, y, z: in Std_Logic);

end or3_gate;

architecture Dataflow of or3_gate is

begin

w <= x or y or z;

end Dataflow;

entity and2_gate is

port (w: out Std_Logic; x, y: in Std_Logic);

end and2_gate;

architecture Dataflow of and2_gate is

begin

w <= x and y;

end Dataflow;

entity and3_gate is

port (w: out Std_Logic; x, y, z: in Std_Logic);

end and 3_gate;

architecture Dataflow of and3_gate is

begin

w <= x and y and z;

end Dataflow;

entity not_gate is

port (x: in Std_Logic; y: out Std_Logic);

end not_gate;

architecture Dataflow of not_gate is

begin

y <= not x;

end Dataflow;

entity Circuit_of_Fig_4_2 is

port (A, B, C: in Std_Logic; F1, F2: out Std_Logic;);

end Circuit_of_Fig. 4_2;

The components are instantiated and connected (byname) to form the circuit:

architecture Structural of  Circuit_of_Fig_4_2 is

signal: T1, T2, T3, F2_b, E1, E2, E3: Std_Logic;

component  or2_gate 	port (w: out Std_Logic; x, y: in Std_Logic);

component  or3_gate 	port (w: out Std_Logic; x, y, z: in Std_Logic);

component and2_gate 	port (w: out Std_Logic; x, y: in Std_Logic);

component and3_gate	port (w: out Std_Logic; x, y, z: in Std_Logic);

component not_gate	port (x: in Std_Logic; y: out Std_Logic);

begin

G1: or3_gate 		port map (w => T1, x => A, y => B, z => C);

G2: and3_gate 		port map (w => T2, x => A, y => B, z => C);

G3: and2_gate 		port map (w => E1, x => A, y => B);

G4: and2_gate 		port map (w => E2, x => A, y => C);

G5: and2_gate 		port map (w => E3, x => B, y => C);

G6: or3_gate 		port map (w => F2, x => E1, y => E2, z => E3)'

G7: not_gate 		port map (x => F2, y => F2_b);

G8: and2_gate 		port map (w => T3, x => T1, y => F2_b);

G9: or2_gate 		port map (w => F1, x => T2, y => T3);

end Structural;

entity t_ Circuit_of_Fig_4_2 is

port ();

end t_ Circuit_of_Fig_4_2;

Finally, Test_Bench, the architecture of  t_Circuit_of_Fig_4_2, is declared. Within it, Circuit_of_Fig_4_2 is declared as a component and instantiated. The signals of its port are connected by name to the stimulus and outputs that were declared locally within Test_Bench.

architecture Test_Benchl of t_Circuit_of_Fig_4_2 is

signal t_A, t_B, t_C: Std_Logic;

signal t_F1, t_F2: Std_Logic;

integer k range 0 to 7: 0;

component Circuit_of_Fig_4_2 port (A, B, C: in Std_Logic;  F1, F2: out Std_Logic);

-- UUT is a component

begin

-- Instantiate (by name) the UUT

UUT: Circuit_of_Fig_4_2 port map (F1 => t_F1, F2 => t_F2, A => t_A, B => t_B, C => t_C);

-- Apply stimulus signals

t_A & t_B & t_C <= '000';

while k <= 7 loop

t_A & t_B & t_C  <= t_A & t_B & t_C   + '001';

k := k + 1;

end loop;

end Test_Bench;