// Structural description of a 4-bit universal shift register (see Fig. 6.7)

module Shift_Register_4_str (		// V2001, 2005

output [3: 0] 	A_par,	/	/ Parallel output

input [3: 0] 	I_par,		// Parallel input

input	s1, s0,	// Mode select

input		MSB_in, LSB_in, CLK, Clear_b		// Serial inputs, clock, clear

);

// bus for mode control

wire [1:0] select = {s1, s0};

// Instantiate the four stages

stage ST0 (A_par[0], A_par[1], LSB_in, I_par[0], A_par[0], select, CLK, Clear_b);

stage ST1 (A_par[1], A_par[2], A_par[0], I_par[1], A_par[1], select, CLK, Clear_b);

stage ST2 (A_par[2], A_par[3], A_par[1], I_par[2], A_par[2], select, CLK, Clear_b);

stage ST3 (A_par[3], MSB_in, A_par[2], I_par[3], A_par[3], select, CLK, Clear_b);

endmodule

// One stage of shift register

module stage (i0, i1, i2, i3, Q, select, CLK, Clr_b);

input	i0,	// circulation bit selection

i1,	// data from left neighbor or serial input for shift-right

i2,	// data from right neighbor or serial input for shift-left

i3;	// data from parallel input

output	Q;

input [1: 0]	select;	// stage mode control bus

input	CLK, Clr_b;	// Clock, Clear for flip-flops

wire	mux_out;

// instantiate mux and flip-flop

wire Clr = ~Clr_b		// Flip-flop has active-high clear signal

// but circuit has active-low clear action

Mux_4x1	M0	(mux_out, i0, i1, i2, i3, select);

D_flip_flop	M1	(Q, mux_out, CLK, Clr);

endmodule

// 4x1 multiplexer	// behavioral model

module Mux_4x1 (mux_out, i0, i1, i2, i3, select);

output	mux_out;

input	i0, i1, i2, i3;

input [1: 0]	select;

reg	mux_out;

always @ (select, i0, i1, i2, i3)

case (select)

2'b00:	mux_out = i0;

2'b01:	mux_out = i1;

2'b10:	mux_out = i2;

2'b11:	mux_out = i3;

endcase

endmodule

// Behavioral model of D flip-flop

module D_flip_flop (Q, D, CLK, Clr);

output	Q;

input	D, CLK, Clr;

reg	Q;

always @ (posedge CLK, posedge Clr)

if (Clr) Q <= 1'b0; else Q <= D;

endmodule