// Test bench for the binary multiplier

module t_Sequential_Binary_Multiplier;

parameter		dp_width = 5;	   // Set to width of datapath

wire 		[2*dp_width -1: 0]		Product;	// Output from multiplier

wire		Ready;

reg 	[dp_width -1: 0]	Multiplicand, Multiplier; // Inputs to multiplier

reg 	Start, clock, reset_b;

// Instantiate multiplier

Sequential_Binary_Multiplier M0 (Product, Ready, Multiplicand, Multiplier, Start, clock,  reset_b);

// Generate stimulus waveforms

initial #200 $finish;

initial

begin

Start = 0;

reset_b = 0;

#2 Start = 1; reset_b = 1;

Multiplicand = 5'b10111;  Multiplier = 5'b10011;

#10 Start = 0;

end

initial

begin

clock = 0;

repeat (26) #5 clock = ~clock;

end

// Display results and compare with Table 8.5

always @ (posedge clock)

$strobe ("C=%b A=%b Q=%b P=%b time=%0d",M0.C,M0.A,M0.Q,M0.P, $time);

endmodule

/*

Simulation log:

C=0 A=00000	Q=10011	P=101 time=5

C=0 A=10111	Q=10011	P=100 time=15

C=0 A=01011	Q=11001	P=100 time=25

C=1 A=00010	Q=11001	P=011 time=35

C=0 A=10001	Q=01100	P=011 time=45

C=0 A=10001	Q=01100	P=010 time=55

C=0 A=01000	Q=10110	P=010 time=65

C=0 A=01000	Q=10110	P=001 time=75

C=0 A=00100	Q=01011	P=001 time=85

C=0 A=11011	Q=01011	P=000 time=95

C=0 A=01101	Q=10101	P=000 time=105

C=0 A=01101	Q=10101	P=000 time=115

C=0 A=01101	Q=10101	P=000 time=125

*/

/* Test bench for exhaustive simulation

module t_Sequential_Binary_Multiplier;

parameter	dp_width = 5;	// Width of datapath

wire	[2 * dp_width -1: 0]	Product;

wire	Ready;

reg 	[dp_width -1: 0]	Multiplicand, Multiplier;

reg 	Start, clock, reset_b;

Sequential_Binary_Multiplier M0 (Product, Ready, Multiplicand, Multiplier, Start, clock, reset_b);

initial #1030000 $finish;

initial begin clock = 0; #5 forever #5 clock = ~clock; end

initial fork

reset_b = 1;

#2 reset_b = 0;

#3 reset_b = 1;

join

initial begin #5 Start = 1; end

initial begin

#5 Multiplicand = 0;

Multiplier = 0;

repeat (32) #10 begin Multiplier = Multiplier + 1;

repeat (32) @ (posedge M0.Ready) #5 Multiplicand = Multiplicand + 1;

end

end

endmodule

*/