Problem Statement – 2

Automated RAM Sequencer

Aim

To design and implement a digital circuit in Logisim that utilizes a counter to sequentially address a block of RAM and store pseudo-random data into those locations.

Concepts Covered

- Sequential Logic (Counters)

- Memory Addressing and Operations (RAM)

- Clocking and Timing (Clocks, Splitters, Wires)

- Data Generation (Random value/Noise Generator)

### Required Components

- Clock: To sequence the operation.

  - Clock A → connected to the Counter and Random Generator (configured to update twice per RAM cycle).

  - Clock B → connected to the RAM (drives the actual write event).

- Counter: To generate sequential memory addresses.

- RAM: 8-bit Data, 16-bit Address.

- Random Generator: A component that outputs a random value (we will use this for the data input).

- Splitter: To manage bus widths

- Power: To provide a continuous logical high value (1) to a circuit, similar to a power supply pin in real electronics.

- Constant/Ground: To set control signals like Load/Clear.

Setup

- Add a RAM unit (available under the "Memory" group) to the simulation area.

- Configure it to have 16 address bits, and 8 data bits, and set the Data Interface property to “Separate load and store ports” (this simplifies the wiring).

- Add a Counter and connect its output to the address bus of the RAM. The counter should have a 16-bit width, and it will increment on every clock pulse.

- Add a Random Generator and connect its output to the Store Data input (D) of the RAM. The Random Generator should have 8 data bits.

- Ground the clear inputs of both the Counter and the Random Generator by connecting a Ground (from the Wiring group) to their respective clear pins. Hover over each component to identify the clear pins.

- Insert two clocks:

  - Clock A – Used for the Counter and Random Generator. Set its High Duration and Low Duration to 2 ticks.

  - Clock B – Used for the RAM (keep its properties at the default values).

- Place two 8-bit Hex Displays side by side to the right of the RAM.

- Use a Splitter to connect the RAM’s data output to both Hex Displays. Connect output bit 0 of the Splitter to the units Hex Display and output bit 1 to the tens Hex Display.

- Use a Power source to provide continuous logical HIGH values. Connect it to the Count input of the Counter, the Enable input of the Random Generator, and the sel (select) pin of the RAM.

When you finish this and run the simulation (ctrl+k), you should see the position in the RAM proceeding and filling it in with random values.

# Expected behaviour / common fixes

- If the RAM isn’t being filled:

  - Confirm st is actually 1 (connected to Power), RAM clock is connected, and address and data lines are multi-bit wires of correct widths.

  - Ensure Counter output matches the RAM address width (use splitter if needed).

  - Make sure Random Generator width = 8 and is actually producing values (watch its display).

- If all memory addresses have the same value:

  - Data did not update between writes → increase Clock A speed relative to RAM clock (so Random Generator updates prior to the RAM write edge).

- If the counter is resetting unexpectedly:

  - Check any clear/reset pins — they should be tied to Ground (0) unless intentionally used.

# Paste the screenshots of the simulation below, showing how the counter sequentially generates memory addresses and how these addresses and corresponding data values are displayed on the hex displays.

# Observation questions

- When you start the simulation, which RAM addresses are written first?

- How many bits wide is the RAM data word and why did you set the Random Generator to that width?

- What happens if you tie the RAM st pin to Ground instead of 1?

- Experiment: change Clock A to run at the same speed as Clock B. What happens to the stored values?

- Why is the frequency of Clock A ticks set to be twice the rate (or 2 ticks per RAM cycle) of Clock B?