Problem Statement – 3

RAM Parity Decoder

### Aim

Design and simulate a Logisim circuit that writes random values into RAM, then reads them back and then identifies the parity (even/odd) of each stored number when reading through it.

Concepts Covered

- Sequential Logic (Counters)

- Memory Addressing and Operations (RAM)

- Clocking and Timing (Clocks, Splitters, Wires)

- Data Generation (Random value/Noise Generator)

- Parity detection (even/odd using LSB)

### Required Components

- Clock: To sequence the operation.

- Counter: To generate sequential memory addresses.

- RAM: 8-bit Data, 16-bit Address.

- Random Generator: A component that outputs a random value (we will use this for the data input).

- Splitter: To manage bus widths

- Power: To provide a continuous logical high value (1) to a circuit, similar to a power supply pin in real electronics.

- Multiplexer (MUX): 2:1, to select between the “even” symbol and the “odd” symbol for the hex display.

- Constants: two constant 4-bit values to feed the MUX inputs (e.g., 0x1 and 0xA).

Setup

- Add a RAM unit (available under the Memory group) to the simulation area.

- Configure it to have 16 address bits, and 8 data bits, and set the Data Interface property to “Separate load and store ports” (this simplifies the wiring).

- Add a Counter and connect its output to the address bus of the RAM. The counter should have a 16-bit width, and it will increment on every clock pulse.

- Add a Random Generator and connect its output to the Store Data input (D) of the RAM. The Random Generator should have 8 data bits.

- Add a Button control and connect it to the clear pin of our counter (this is an async reset). Hint: to test this is working switch to the finger curser and press this button, you should see the counter reset to zero immediately. Hover over each component to identify the clear pins.

- Ground the clear input of the Random Generator by connecting a Ground (from the Wiring group) to the respective clear pin. Hover over each component to identify the clear pins.

- Insert a Clock and connect it to all clock inputs (RAM, Counter, and Random Generator).

- Add a Pin control (under the Wiring group) with its "three state" and "output" properties set to ‘No’. Connect its output to the ld (load) pin of the RAM, and the NOT of its output (using a NOT gate) to the st (store) pin of the ram. Hint: to test this is working switch to the finger curser and press the pin to toggle it between high and low. This pin will default to low enabling writes, and we will switch it to high to read from the RAM. We will call this the "read/write pin".

- Add a Button control and connect it to the clr (clear) pin of the RAM. This will be used to clear the address content stored in RAM.

- From the right side of the RAM, you’ll see an 8-bit data output bus (labeled D). Use a splitter (from the Wiring group) to extract the lowest bit (bit 0 - the least significant bit).

- Set the splitter’s bit width to 8. Set fan-out to 8.

- Connect the combined end of the splitter to the output bus of RAM. Connect the bit 0 output of the splitter to the MUX select input. (Select Multiplexer from the Plexers group and connect the bit 0 of splitter to the select pin – indicated by a dot on the MUX)

- Set the data bits of the multiplexer to 4.

- Place two constant components (from Wiring group - Constant). Set both to 4 bits wide (you can change this in the constant’s properties). Also set the values as follows:

  - Constant 1 → 0xe (hex e) - connect this to input 0 of MUX

  - Constant 2 → 0xa (hex A) - connect this to input 1 of MUX

- Place a Hex Display (from Input/Output group). Connect the MUX output directly to the Hex Display input.

- Use a Power source to provide continuous logical HIGH values. Connect it to the Count input of the Counter, the Enable input of the Random Generator.

- Use another Power source and connect it to the Enable pin of the Multiplexer.

Simulation Procedure

- Click on Simulate on the menu bar, and click on Simulation Enabled.

- Then click on Ticks Enabled (or simply Ctrl + K) to start the simulation.

- Make sure the read/write pin of the RAM is set to 0.

- The counter starts stepping through the addresses, and we can see the addresses being stored in RAM. Allow this to happen for approximately 2 minutes. (or unless you have enough values stored in RAM)

- After this, turn off the ticks. (Click on Ticks Enabled again, or simply Ctrl + K again). You see values stored in the RAM now.

- Click on the Reset Counter Button. (The button we initially connected to the clear pin of the COUNTER). This moves the selected address to the first address of the RAM.

- Now switch the read/write pin of the RAM to 1 (high).

- Start the simulation again, i.e., click on Ticks Enabled under Simulation in Menu (or Ctrl + K)

- You will see outputs on the hex display which are being toggled by the Multiplexer. The display switches between E and A as the counter reads through memory.

### Expected Behaviour

- During writing: RAM fills with random values across addresses.

- During reading: MUX toggles between two display values based on parity (even or odd).

- The Hex Display will continuously change as the counter steps through RAM. Note down which value (E or A) appears when the stored number is even - this identifies your parity mapping.

### Common Fixes

- If nothing changes on display: check if the read/write pin is set to 1 (high).

- If RAM doesn’t store: verify store (st) and load (ld) control logic.

- If counter isn’t resetting: ensure button is connected to clr, not ld.

- If MUX isn’t switching: check connection from splitter bit 0 (lowest bit) to MUX select pin.

Attach the following screenshots in your submission:

- While writing to RAM (values being stored).

- During reading phase - when display shows even value.

- During reading phase - when display shows odd value.

### Observation Questions

- Which constant value (0xE or 0xA) corresponds to even parity? How did you determine this?

- What would happen if you forgot to reset the counter before switching to read mode?

- If you swap the MUX inputs, what change do you observe on the Hex Display output?

- Why is the LSB used for parity detection in this design?