library ieee;

use ieee.std_logic_1164.all;

-- Model for 2-input AND component

entity and2_gate is

port (A, B: in Std_Logic; C: out Std_Logic);

end and2_gate;

architecture Boolean_Equation of and2_gate is

begin

C <= A and B;	-- Logic operator

end Boolean_Equation;

-- Model for 2-input OR component

entity or2_gate is

port (A, B: in Std_Logic; C: out Std_Logic);

end or2_gate;

architecture Boolean_Equation of or2_gate is

begin

C <=  A or B;	-- Logic operator

end Boolean_Equation;

-- Model for exclusive-or component

entity xor_2_gate is

port (A, B: in Std_Logic; C: out Std_Logic);

end xor_2_gate;

architecture Boolean_Equation of xor_2_gate is

begin

C <= A xor B;

end Boolean_Equation;

The components and2_gate and xor2_gate are then used in models for Add_half_vhdl and Add_full_vhdl.

entity  Add_half_vhdl is

port (a, b: in std_logic; c_out, sum: out std_logic);

end Add_half

architecture Structure of Add_half is

component and2_gate			-- Identify component being used

port (a, b: in std_logic; c: out std_logic);	-- Identify port of the component

end component;

component xor2_gate	-- Component declaration

port (a, b: in std_logic; c: out std_logic);

and component;

begin			-- Instantiate components and connect ports

G1: xor2_gate	port map (a, b, sum);

G2: and2_gate	port map (a, b, c_out,);

end Structure;

entity Add_full_vhdl is

port (a, b, c_in: in std_logic; c_out, sum: out std_logic);

end Add_full_vhdl

architecture Structure of Add_full_vhdl is

component or2_gate

port (a, b: in std_logic; c: out std_logic);

end component

component Add_half_vhdl

port (a, b: in std_logic; c_out, sum: out std_logic);

end component;

signal w1, w2, w3: std_logic;

begin

M0: Add_half_vhdl port map (b, c_in, c_out, sum);

M1 Add_half port map (a, b, w1, w2);

G1 or2_gate port map (w1, w3, c_out);

end Structure;

entity Add_rca_4_vhdl is

port (A, B: in bit_vector (3 downto 0); c_in: in Std_Logic;

c_out: out Std_Logic; sum: out bit_vector (3 downto 0);

end Add_rca_4_vhdl;

architecture Structure of Add_rca_4_vhdl is

component Add_full_rca_vhdl

port (a, b, c_in: in Std_Logic; c_out, sum: out Std_Logic);

end component;

signal c_in1, c_in2, c_in3;

begin

M0: Add_full_vhdl port map (a(0), b(0), c_in,   c_in1, sum(0));

M1: Add_full_vhdl port map (a(1), b(1), c_in1, c_in2, sum(1));

M2: Add_full_vhdl port map (a(2), b(2), c_in2, c_in3, sum(2));

M3: Add_full_vhdl port map (a(3), b(3), c_in3, c_out, sum(3));

end Structure;

entity Add_rca_8_vhdl is

port (a, b: in Std_Logic_Vector_vector (7 downto 0); c_in: in Std_Logic_Vector;

c_out: out Std_Logic_Vector, sum: Std_Logic_Vector (7 downto 0));

end Add_rca_8_vhdl;

architecture Structure of Add_rca_8_vhdl is

component Add_rca_4_vhdl;

port (a, b: in Std_Logic_Vector (3 downto 0); c_in: in Std_Logic,

c_out: out Std_Logic, sum: Std_Logic_Vector (3 downto 0));

end component;

signal c_in4		-- Connects 4-bit adders

M0 Add_rca_4_vhdl port map (a(3 downto 0), b(3 downto 0), c_in, c_in4, sum(3 downto 0 ));

M1 Add_rca_4_vhdl port map (a(7 downto 4), b(7 downto 4), c_in4, c_out, sum(7 downto 4 ));

end Structure