library ieee;

use ieee.std_logic_1164.all;

-- Declare entity-architecture pairs that will be components

-- Model for inverter component

entity inv_gate is

port (A: in std_logic; B: out std_logic);

end inv_gate;

architecture Boolean_Equation of inv_gate is

begin

B <= not A;

end Boolean_Equation;

entity nand2_gate is

port (C: out std_logic; A, B: in std_logic);

end nand2_gate;

architecture Boolean_Eq of nand2_gate

begin

C <= not (A and B);

end Boolean_Eq;

-- Gate-level description of two-to-four line decoder

entity decoder_2x4_gates_vhdl is

port (A, B, enable: in std_logic; D: out std_logic_vector range 0 to 3);

end decoder_2x4_gates_vhdl;

architecture Structure of  decoder_2x4_gates_vhdl is

-- Identify components and ports

component inv_gate

port (B: out std_logic; A: in std_logic);

end component;

component nand2_gate

port (C: out std_logic; A, B: in std_logic);

end component;

signal A_not, B_not, enable_not;	-- Internal signals

begin	-- Instantiate components and connect ports via port maps

G1: inv_gate port map (A_not, A);

G2: inv_gate port map (B_not, B);

G3: inv_gate port map (enable_not, enable);

G4: nand_gate port map (D(0), A_not, B_not, enable_not);

G5: nand_gate port map (D(1), A_not, B, enable_not);

G6: nand_gate port map (D(2), A, B_not, enable_not);

G7: nand_gate port map (D(3), A, B, enable_not);

end Structure