entity half_adder_vhdl is

port (S, C: out Std_Logic; x, y: in Std_Logic);

end half_adder_vhdl;

architecture Dataflow of half_adder_vhdl is

S <= x xor y;

C <= x and y;

end Dataflow;

entity full_adder_vhdl is

port (S, C: out Std_Logic; x, y, z: in Std_Logic);

end half_adder_vhdl

architecture Structural of full_adder_vhdl is

signal S1, C1, C2: Std_Logic;

component half_adder_vhdl (S, C, port x, y, z);

begin

HA1: half_adder_vhdl port map (S => S1, C => C1, x => x, y => y);

HA2: half_adder_vhdl port map (S => S, C => C2, x => S1, y => z);

C <= C2 or C1;

end Structural;

entity ripple_carry_4_bit_adder_vhdl is

port (Sum: out Std_Logic_Vector (3 downto 0); C4: out Std_Logic; A, B: in

Std_Logic_Vector (3 downto 0); C0: in Std_Logic);

end ripple_carry_4_bit_adder_vhdl;

architecture Structural of ripple_carry_4_bit_adder_vhdl is

port (S3 downto 0); Sum: out Std_Logic_Vector (3 downto 0); C4: out Std_Logic; A, B: in Std_Logic_Vector (3 downto 0); C0:in Std_Logic);

signal C1, C2, C3: Std_Logic;

component full_adder_vhdl (S, C, x, y, z);

begin

FA0: full_adder_vhdl port map (S => Sum(0), C => C1, x => A(0), y = B(0), z => C0);

FA1: full_adder_vhdl port map (S => Sum(1), C => C2, x => A(1), y = B(1), z => C1);

FA2: full_adder_vhdl port map (S => Sum(2), C => C3, x => A(2), y = B(2), z => C2);

FA3: full_adder_vhdl port map (S => Sum(3), C => C4, x => A(3), y = B(3), z => C3);

end ripple_carry_4_bit_adder_vhdl;