// Gate-level description of two-to-four-line decoder

// Refer to Fig. 4.19 with symbol E replaced by enable, for clarity.

module decoder_2x4_gates (D, A, B, enable);

output [0: 3]	D;

input		A, B;

input		enable;

wire		A_not, B_not, enable_not;

not

G1  (A_not, A),	// Comma-separated list of primitives

G2  (B_not, B),

G3  (enable_not, enable);

nand

G4  (D[0], A_not, B_not, enable_not),

G5  (D[1], A_not, B, enable_not),

G6  (D[2], A, B_not, enable_not),

G7  (D[3], A, B, enable_not);

endmodule