// RTL description of design example (see Fig. 8.11)

module Design_Example_RTL (A, E, F, Start, clock, reset_b);

// Specify ports of the top-level module of the design

// See block diagram, Fig. 8.10

output [3: 0] A;

output	E, F;

input	Start, clock, reset_b;

// Instantiate controller and datapath units

Controller_RTL M0 (set_E, clr_E, set_F, clr_A_F, incr_A, A[2], A[3], Start, clock, reset_b);

Datapath_RTL M1 (A, E, F, set_E, clr_E, set_F, clr_A_F, incr_A, clock);

endmodule

module Controller_RTL (set_E, clr_E, set_F, clr_A_F, incr_A, A2, A3, Start, clock, reset_b);

output reg	set_E, clr_E, set_F, clr_A_F, incr_A;

input	Start, A2, A3, clock, reset_b;

reg [1: 0] 	state, next_state;

parameter 	S_idle = 2'b00, S_1 = 2'b01, S_2 = 2'b11;	// State codes

always @ (posedge clock, negedge reset_b)	// State transitions (edge sensitive)

if (reset_b == 0) state <= S_idle;

else state <= next_state;

// Code next-state logic directly from ASMD chart (Fig. 8.9d)

always @ (state, Start, A2, A3) begin	// Next-state logic (level sensitive)

next_state = S_idle;

case (state)

S_idle:	if (Start) next_state = S_1; else next_state = S_idle;

S_1: 	if (A2 & A3) next_state = S_2; else next_state = S_1;

S_2: 	next_state = S_idle;

default:	next_state = S_idle;

endcase

end

// Code output logic directly from ASMD chart (Fig. 8.9d)

always @ (state, Start, A2) begin

set_E   = 0;	// default assignments; assign by exception

clr_E  = 0;

set_F   = 0;

clr_A_F = 0;

incr_A = 0;

case (state)

S_idle: 	if (Start) clr_A_F = 1;

S_1: 	begin incr_A = 1; if (A2) set_E = 1; else clr_E = 1; end

S_2: 	set_F = 1;

endcase

end

endmodule

module Datapath_RTL (A, E, F, set_E, clr_E, set_F, clr_A_F, incr_A, clock);

output reg [3: 0] A;	// register for counter

output reg 	E, F;	// flags

input 	set_E, clr_E, set_F, clr_A_F, incr_A, clock;

// Code register transfer operations directly from ASMD chart (Fig. 8.9(d))

always @ (posedge clock) begin

if (set_E) 	E <= 1;

if (clr_E) 	E <= 0;

if (set_F) 	F <= 1;

if (clr_A_F)	begin A <= 0; F <= 0; end

if (incr_A) 	A <= A + 1;

end

endmodule