A small IoT temperature sensor microcontroller converts an analog voltage to a 4-bit ADC output.
The output represents temperature × 3 (scaled).
To obtain the actual temperature in °C, the controller must divide the ADC reading by 3 using a binary division circuit.

Perform the operation using both Restoring and Non-Restoring Division algorithms for the following data:

- Dividend (ADC output) = 1101₂ (13₁₀)

- Divisor (Scaling constant) = 0011₂ (3₁₀)

Tasks:

- Perform the Restoring Division step-by-step.

- Perform the Non-Restoring Division step-by-step.

- Show partial remainders and quotient bits per cycle.

- Compare the total number of arithmetic (add/subtract) operations in both methods.

- Conclude which algorithm is more energy-efficient for IoT nodes and why.