LED Brightness Booster

A wearable health band drives a tiny RGB LED. The firmware wants to increase the RED channel brightness by multiplying the current 4-bit signed intensity by a gain of –2 (to invert and double in a single step).

- Current RED intensity (signed 4-bit two’s complement):(+6)

- Gain factor (signed 4-bit two’s complement): (–2)

Tasks:

- Write down (Q₀, Q₋₁) pairs for the multiplier according to Booth’s rule.
Show the sequence of operations on the accumulator: Add M / Sub M / Shift right, cycle by cycle.

- Give the final 8-bit product in binary and decimal.

- Briefly state why “–2 × 6” doesn’t need repeated subtraction if Booth is used.

Ans:

Convert to 4-bit signed (two’s complement)

+6 → 0110
–2 → 1110

We'll treat:

- M = 0110 (this is +6)

- Q = 1110 (this is –2)

- We also keep an extra bit Q₋₁, initialized to 0

- We use an accumulator A (starts 0000)

- We'll run for n = 4 cycles because Q is 4 bits

Booth's rules for (Q₀, Q₋₁) at each cycle are (from your slides):

- (0,0) or (1,1) → do nothing (no add/sub)

- (0,1) → ADD M to A

- (1,0) → SUB M from A

After doing the add/sub (if any), we perform an arithmetic right shift of [A,Q,Q₋₁] together.

We’ll track in a table.

Initial:

- A = 0000

- Q = 1110

- Q₋₁ = 0

- M = 0110

We'll also need −M when subtracting:
M = 0110 (+6)
Two’s complement of M = 1010 (which is −6)

Cycle 1

Current bits:

- Q₀ = least significant bit of Q = 0 (Q = 1110)

- Q₋₁ = 0
→ Pair (Q₀, Q₋₁) = (0,0)

Booth rule (0,0): do nothing to A.

So before shift:

- A = 0000

- Q = 1110

- Q₋₁ = 0

Now do arithmetic right shift of [A,Q,Q₋₁] as one 9-bit bundle.
[A,Q,Q₋₁] = 0000 1110 0

Arithmetic right shift:

- Shift everything right by 1

- The sign bit of A (leftmost bit of A) is 0, so we shift in 0 on the left

0000 1110 0 >> 1 → 0000 0111 0

After shift:

- A = 0000

- Q = 0111

- Q₋₁ = 0

Cycle 2

Now:

- Q = 0111 → Q₀ = 1

- Q₋₁ = 0
Pair (1,0)

Booth rule (1,0): SUBTRACT M
That means: A = A − M = A + (−M)

We said:

- A = 0000

- −M = 1010 (−6)

Add:
0000
+1010
=1010

So after operation:

- A = 1010 (this is −6 in two’s complement)

Now shift [A,Q,Q₋₁]:
Current [A,Q,Q₋₁] before shift = 1010 0111 0

Arithmetic right shift by 1:
Since A is negative now (A's sign bit is 1), we replicate 1 on the left.

1010 0111 0 >> 1 → 1101 0011 1

After shift:

- A = 1101

- Q = 0011

- Q₋₁ = 1

Cycle 3

Now:

- Q = 0011 → Q₀ = 1

- Q₋₁ = 1
Pair (1,1)

Booth rule (1,1): do nothing.

So A remains 1101.

Shift [A,Q,Q₋₁]:
[A,Q,Q₋₁] before shift = 1101 0011 1

Arithmetic right shift by 1:
Sign bit is 1 (A is negative), so shift in 1 on the left.

1101 0011 1 >> 1 → 1110 1001 1

After shift:

- A = 1110

- Q = 1001

- Q₋₁ = 1

Cycle 4 (final cycle)

Now:

- Q = 1001 → Q₀ = 1

- Q₋₁ = 1
Pair (1,1)

Booth rule: do nothing again.

Now shift one last time:
[A,Q,Q₋₁] = 1110 1001 1

Arithmetic right shift:
Sign bit is 1 → shift in 1

1110 1001 1 >> 1 → 1111 0100 1

After final shift:

- A = 1111

- Q = 0100

- Q₋₁ = 1

We have completed 4 cycles (equal to the bit-width of Q).
The final product is the concatenation of A and Q:
Product = A‖Q = 1111 0100

That is an 8-bit signed result.

Step 3. Interpret the result

Binary product:
11110100₂

Check the decimal value:

- We expected (+6) × (−2) = −12.

Now verify 11110100₂ is −12 in 8-bit two’s complement:

- Take two’s complement to find magnitude:

  - Invert 11110100 → 00001011

  - Add 1 → 00001100 = 12₁₀

- Original had leading 1 → negative.

So 11110100₂ = −12.

This matches math:
(+6) × (−2) = −12.

So:

Answer to Task 2:
Final 8-bit product = 11110100₂
Decimal = −12

Step 4. Sequence summary table (Tasks 1 & 2 together)

| Cycle | Q₀Q₋₁ | Booth action on A | A before shift | [A,Q,Q₋₁] after shift |
| --- | --- | --- | --- | --- |
| 1 | 00 | No op | A=0000 | 0000 0111 0 |
| 2 | 10 | A = A − M (subtract M) | A=1010 | 1101 0011 1 |
| 3 | 11 | No op | A=1101 | 1110 1001 1 |
| 4 | 11 | No op | A=1110 | 1111 0100 1 |

Final product = A‖Q = 1111 0100.

Why we don’t need repeated subtraction (Task 3)

If you multiply 6 × (−2) in a naïve hardware multiplier, you'd conceptually do:
-2 + -2 + -2 + -2 + -2 + -2 (six times),
or you'd generate six partial products and add/sub them all.

That’s a lot of ALU activity.

Booth’s trick:

- It looks at pairs of bits in the multiplier (Q₀, Q₋₁).

- A long run like ...1100... doesn't mean “subtract M many times.”
It becomes a single “subtract M once here” plus shifts.

- In our case, only one real arithmetic update (the subtract in Cycle 2) was needed to encode the effect of multiplying by −2.

Why this matters in hardware (wearable / LED driver / DSP chip):

- Every add/sub toggles a lot of transistors in the adder.

- Fewer add/sub steps → less switching → lower dynamic power.

- Lower dynamic power = longer battery life.

This is exactly why Booth’s multiplier is taught as “reduces number of partial products / reduces number of add & sub” in your slides.