An IoT weather station continuously reads analog temperature values using a 12-bit ADC.
To calibrate readings for cloud upload, the microcontroller internally converts each value into a 32-bit IEEE-754 floating-point format (single precision).

During testing, the system outputs the hexadecimal floating-point word 0xC2690000.

Tasks

- Interpret the 32-bit IEEE-754 word
Decode the given hexadecimal value step by step to obtain:

  - Sign bit

  - Biased exponent and actual exponent

  - Mantissa bits

  - Final real (decimal) value of the temperature reading.

- Explain the internal representation
Show how the value is normalized in binary scientific notation of the form

- Application Insight

  - What real-world temperature (in °C) does this correspond to?

  - If the system used fixed-point (4 integer + 4 fractional bits) instead of floating-point, explain briefly how the range and precision would change for sensor calibration.

- Suppose the sensor produced an underflow reading.

  - What range of exponent values would indicate an underflow in single-precision IEEE-754 format?

  - How does the system typically handle such values (e.g., subnormal numbers or zero)?

Ans:

| Step | Operation | Explanation |
| --- | --- | --- |
| 1 | Hex → Binary | 0xC2690000 = 1100 0010 0110 1001 0000 0000 0000 0000 |
| 2 | Sign bit | s = 1 → negative |
| 3 | Exponent | 10000100₂ = 132 → Actual exponent E = 132 – 127 = 5 |
| 4 | Mantissa | 11010010000000000000000₂ → 1.1101001₂ = 1.828125 |
| 5 | Value | (–1)¹ × 1.828125 × 2⁵ = –58.25 |
| 6 | Interpretation | Sensor’s internal floating-point output corresponds to –58.25°C |
| 7 | Insight | Floating-point allows representing negative subzero temperatures accurately; fixed-point would overflow or lose precision for such small values. |
| 8 | Underflow | When exponent bits = 00000000 (E_actual = –126) and mantissa ≠ 0 → denormalized (subnormal) range. |