An IoT weather station continuously reads analog temperature values using a 12-bit ADC.
To calibrate readings for cloud upload, the microcontroller internally converts each value into a 32-bit IEEE-754 floating-point format (single precision).

During testing, the system outputs the hexadecimal floating-point word 0xC2690000.

Tasks

- Interpret the 32-bit IEEE-754 word
Decode the given hexadecimal value step by step to obtain:

  - Sign bit

  - Biased exponent and actual exponent

  - Mantissa bits

  - Final real (decimal) value of the temperature reading.

- Explain the internal representation
Show how the value is normalized in binary scientific notation of the form

- Application Insight

  - What real-world temperature (in °C) does this correspond to?

  - If the system used fixed-point (4 integer + 4 fractional bits) instead of floating-point, explain briefly how the range and precision would change for sensor calibration.

- Suppose the sensor produced an underflow reading.

  - What range of exponent values would indicate an underflow in single-precision IEEE-754 format?

  - How does the system typically handle such values (e.g., subnormal numbers or zero)?